Recommended WiFi Network Configuration

Connectivity Requirements

- WiFi Standard: WiFi 4 (802.11 b/g/n) or WiFi 6 (802.11 ax)

- Frequency Band: 2.4 GHz (mandatory)

- Supported Protocols: b/g/n or ax

- Security: WPA2 enabled

Advanced Router Configuration

- WMF (Wireless Multicast Forwarding): enabled

- Client Isolation (AP Isolation / Client Isolation): disabled

  - Especially relevant for routers provided by operators such as Movistar and O2

- WPS (WiFi Protected Setup): disabled

ISP Configuration

- The network must not operate under CG-NAT (Carrier-Grade NAT)

  - If CG-NAT is active, a public IP address must be requested from the ISP

  - For example, DIGI offers this option through an additional service package

  - This is one of the most common causes of pairing and remote connectivity issues

Best Practices During Device Pairing

- Temporarily disable the router firewall if the device fails to pair

- Avoid dual-band WiFi networks (2.4 GHz and 5 GHz under the same SSID)

  - It is recommended to use a dedicated SSID for the 2.4 GHz network

- If pairing fails using multicast, use point-to-point mode (AP mode) via the app

SSID and Password Requirements

- SSID (network name):

  - Must not contain spaces

  - Avoid special characters

- WiFi Password:

  - Recommended length: 8 to 15 characters

Router Capacity

- Verify the maximum number of connected devices supported by the ISP router

  - These devices typically have limited capacity

- If additional capacity is required:

  - Install an additional neutral router

  - Connect it via Ethernet to the ISP router

CG-NAT Usage in ISPs (Spain and Italy)

Definition

CG-NAT (Carrier-Grade NAT) is a technology used by some Internet Service Providers to allow multiple users to share a single public IP address.

This can limit essential network functionalities required for home automation and IoT systems.

Impact on IoT Devices

When a connection operates under CG-NAT:

- Port forwarding is not possible

- Remote access to devices is restricted

- Pairing and app control issues may occur

- Integration with third-party systems (e.g., local servers or VPNs) may be affected

ISPs Commonly Using CG-NAT

Spain

- DIGI

- MásMóvil Group (Yoigo, Pepephone, MásMóvil, etc.)

- Adamo

- Finetwork

- Orange / Jazztel (in specific configurations)

Italy

- Iliad

- Fastweb

- EOLO and WISP operators

Solution

To ensure proper system functionality, a public IP address is required.

Procedure

- Contact the ISP

- Request: “Public IP” or “opt-out of CG-NAT”

Considerations

- This may be free of charge or involve an additional cost

- Example: DIGI offers it as an add-on service

Technical Recommendation

Before installation:

- Verify whether the connection uses CG-NAT

- If so, request a public IP

- Confirm remote connectivity after configuration

Technical Support Note

If the device does not allow remote access or experiences communication issues, verify whether CG-NAT is active on the customer’s network, as it is a common root cause.