Technical Specifications for Device Operation and Pairing

1. Factory Mode

Factory Mode refers to the initial state in which devices are delivered to the customer.

- Upon power-up, the device automatically enters a 60-minute pairing window.

- During this period, the device indicates availability through LED blinking every 2 seconds.

- After this time:

  - The device exits pairing mode.

  - It operates exclusively in local switch mode.

  - The LED turns off.

Returning to Factory Mode

To reactivate Factory Mode:

- Press the reset button, or

- Power cycle the device (disconnect and reconnect power).

In both cases, a new 60-minute pairing window is initiated.

2. Reset Button Location by Device

CONNECT-1 (Essential / Wave1 / ICON.1)

- Device with two capacitive buttons.

- Reset button located above the up button, slightly shifted to the right.

- Factory Mode activation: single short press (if not previously paired).

CONNECT-2 (Sentry / Wave2 / ICON.2)

- Device with three capacitive buttons.

- Reset button located within the upper circular area, slightly shifted to the left.

- Factory Mode activation: single short press (if not previously paired).

C-WALL (Pulse / Wave3 / ICON.3)

- To access the reset button:

  - Remove the front cover.

  - Identify the black physical button on the PCB.

- Factory Mode activation: single short press (if not previously paired).

Könect Élite (Sense)

- Located at the bottom of the shutter box, on one side.

- Identification:

  - Circular hole labeled RESET.

  - Do not confuse with the rectangular sensor opening.

- Factory Mode activation: single short press (if not previously paired).

CONNECT PULSAR (Wave4 / ICON MINI)

- Factory Mode activation:

  - Press and hold the button for more than 8 seconds, or

  - Power cycle the device.

CONNECT EVO

- Device with two mechanical buttons.

- Reset button located above the up button, slightly shifted to the right.

- Factory Mode activation: single short press (if not previously paired).

3. Hard Reset (Full Reset)

A Hard Reset restores a previously paired device to Factory Mode.

It can be performed:

- From the mobile app (if the original WiFi network is still available), or

- Via hardware procedure.

General Procedure (CONNECT-1, CONNECT-2, C-WALL, CONNECT EVO)

- Press the reset button (single short press).

- Immediately press and hold simultaneously:

  - Up button

  - Down button

- Hold for 5 seconds.

✔ The device will confirm the reset by blinking the LED every 2 seconds.

Könect Élite (Sense)

- Press the reset button (single short press).

- Fully cover the gesture sensor (very close to the device).

- Hold for at least 5 seconds.

✔ The LED will start blinking green, indicating Factory Mode.

CONNECT PULSAR (Wave4 / ICON MINI)

- Press and hold the button for more than 8 seconds until it starts blinking.

- Release the button.

- After approximately 2 seconds, press again for 5 seconds.

✔ The device enters Factory Mode.

4. Pairing Modes

4.1 Multicast Pairing Mode

- Visual indication:
LED blinking every 2 seconds.

- Allows simultaneous pairing of multiple devices.

4.2 Point-to-Point Pairing Mode

- Visual indication:
LED continuously ON.

- Allows individual device pairing.

Activating Point-to-Point Mode

This can be done by:

- Following the pairing assistant in the mobile app, or

- Pressing and holding both up and down buttons simultaneously while the device is in Factory Mode.

5. LED Status Summary

| LED Status | Meaning |
| --- | --- |
| Blinking every 2 seconds | Factory Mode / Multicast pairing |
| Solid ON | Point-to-point pairing mode |
| OFF | Local switch mode (no active pairing) |