LINKING MANUAL – MySmartWindow DEVICES

1. INTRODUCTION

The linking process allows devices to be connected to the MySmartWindow system for control via the mobile application.
There are three linking methods, each with a different operation, ordered by speed and reliability .

2. TYPES OF LINKING

2.1 MULTI-LINKING (MULTICAST METHOD)

Definition:

Multi-linking is an automatic connection method in which the application sends WiFi network credentials to multiple devices simultaneously through the local network (broadcast/multicast).

Main characteristics:

- Allows linking multiple devices at the same time

- It is the fastest and most automated method

- Requires devices to be in listening mode (multicast mode)

- Strongly dependent on WiFi network quality

How it works:

- Devices are set to multicast mode (blinking every 2 seconds)

- The app sends WiFi credentials to all devices

- Devices receive the information and connect automatically

- Linking is validated and completed

Advantages:

- High efficiency for large installations

- Mass linking in seconds

- Fully automated process

Limitations:

- May fail if the network has:

  - poor coverage

  - client isolation

  - router restrictions

2.2 AP METHOD (POINT-TO-POINT)

Definition:

The point-to-point method consists of a direct connection between the mobile device and the device through a private WiFi network created by the device itself.

Main characteristics:

- Direct connection device ↔ mobile

- Does not initially depend on the router

- More stable in problematic environments

How it works:

- The device enters AP mode

- It creates a private network (e.g., BDSMART)

- The user manually connects to this network

- The app sends the home WiFi credentials

- The device connects to the final network

Advantages:

- Very high reliability

- Works in complex network environments

- Full control of the process

Limitations:

- Slower than multi-linking

- Requires manual user interaction

- Must be done device by device

2.3 APP METHOD / ASSISTED LINKING

Definition:

This is a guided method where the user configures the device step by step through the app assistant, using previously added WiFi networks.

Main characteristics:

- Guided process through the assistant

- Uses WiFi networks previously registered in the app

- Continuous user–app interaction

How it works:

- The user adds the WiFi network in the app

- Selects the linking method from the assistant

- The device enters configuration mode

- The app guides the linking step by step

Advantages:

- Easy for non-technical users

- Visual control of the process

- Fully integrated within the app

Limitations:

- Slower than multicast

- Less efficient for large installations

3. METHOD COMPARISON

| Method | Connection Type | Speed | Reliability | Recommended Use |
| --- | --- | --- | --- | --- |
| Multi-linking | WiFi Broadcast | Very high | Medium | Large installations |
| Point-to-point (AP) | Direct | Medium | Very high | Network issues |
| App Method | Assisted | Medium-low | High | Basic users |

4. CONCLUSION

- Multi-linking → best for speed and scalability

- Point-to-point (AP) → best for reliability

- App method → best for simplicity

The choice of method mainly depends on network quality and the number of devices to be linked.